Western Blotting Method

Hongbao Ma *, Kuan-Jiunn Shieh **

* Department of Medicine, Michigan State University, East Lansing, Michigan, USA.

517-432-0623,  hongbao@msu.edu

** Department of Chemistry, Chinese Military Academy

Fengshan, Kaohsiung, Taiwan 830, ROC

Abstract: Western blotting is a useful and sensitive method to measure the ng/ml to pg/ml ordered materials in the solution, such as serum, urine and culture supernatant. It is especially widely used in protein detection. This article describes the principle technique for Western blotting procedure. [The Journal of American Science. 2006;2(2):23-27].

Keywords: method; protein; SDS-polyacrylamide gel electrophoresis (SDS-PAGE); Western blotting

# 1. Introduction

Western blotting  is widely used in the protein detection. Since the inception of the protocol for protein transfer from an electrophoresed gel to a membrane in 1979, protein blotting has evolved greatly (Kurien, 2006). Western blotting analysis can detect one protein in a solution that contains any number of proteins and giving the protein information (Dechend, 2006; Ma, 1994; 2004; Peter-Katalinic, 2005; Sakudo, 2006; Westermeier, 2005). Western blotting method is normally used with a high-quality antibody directed against a desired protein. First, separate the proteins using SDS-polyacrylamide gel electrophoresis (SDS-PAGE) is the beginning of the Western blotting. This separates the proteins by size. Second, transfer the protein from SDS-gel to a nitrocellulose membrane (electric transfer). Third, put the primary antibody on the membrane. Fourth, use the secondary antibody (this antibody should be an antibody-enzyme conjugate, e.g., horseradish peroxidase (HRP)). Finally, use the dye and read the result. Recently, Knudson et al used Western blotting and ELISA to measure plasma endothelin-1 and it showed that plasma endothelin-1 concentrations were not different between control and prediabetic dogs. Also, they used Western blotting method and revealed a significant decrease in endothelin-A receptor protein in left circumflex coronary arteries (Knudson, 2006). This article describes the principle technique for Western blotting procedure.

- Tissue Sample Preparation:

- Isolate tissue (about 1 gram).

- Put tissue in 3 volume of extract buffer.

- Extract buffer (Table 1): The half-life of a 0.02 mM aqueous solution of PMSF is about 35 minutes at 8.0 pH. PMSF is usually stored as a 10 mM or 100 mM stack solution (1.74 or 17.4 mg/ml in isopropanol) at –20oC.

- Homogenize sample under ice.

- Centrifuge sample at 10,000 rpm for 10 minutes at 4oC.

- Keep supernatant at -70oC until usage.

- SDS PAGE:

- Use 12% SDS gel. 12% SDS gel preparation is shown in Table 2 and an optional 12% SDS gel preparation reagent amount is shown in Table 3.

- Take 50 l of sample and add an equal volume of 2 x SDS gel-loading buffer. 2 x SDS gel-loading buffer is shown in Table 4. 2 x SDS gel-loading buffer lacking dithiothreitol can be stored at room temperature. Dithiothreitol should then be added, just before the buffer is used, from a 1 M stock (Dissolve 3.09 g of dithiothreitol in 20 ml of 0.01 M sodium acetate (pH 5.2). Sterilize by filtration. Dispense into 1-ml aliquots and store at –20oC).

- Boil the sample (in loading buffer) at 100oC for 3 – 5 minutes.

- Load the sample for electrophoresis: 8 V/cm (6 x 8 = 48 volts) before the bromophenol blue (dye) front has moved into the resolving gel and 15 V/cm (6 x 15 = 90 volts) until the bromophenol blue reaches the bottom of the resolving gel.

- Make the gel for transfer in transfer buffer: 0.65 mA/cm2 (about 100 volts) for 1.5 – 2 hours, or 30 volts overnight, on ice.

- Western blotting transfer buffer (Table 5).

- Block the filter with blocking buffer for 1 – 2 hours at room temperature (0.1 ml blocking solution per cm2 filter), with gentle agitation on a platform shaker. Blocking solution is shown in Table 6 and Phosphate-buffered saline (PBS) (pH 7.4, 1000 ml) is shown in Table 7.

- Discard blocking solution and immediately incubate filter with primary antibody.

- Add 10 ml (0.1 ml of blocking solution per cm2 of filter). Blocking solution is shown in Table 8.

- Add 0.005 ml of primary antibody (1:2000) in to blocking solution.

- Incubate at 4oC for 2 hours or overnight with gentle agitation on a platform shaker.

- Discard blocking solution and wash filter 3 times (10 minutes each time) with 250 ml of PBS.

- Incubate the filter with 150 mM NaCl, 50 mM Tris-HCl (pH 7.5) (phosphate-free, azide-free blocking solution) for 3 times for 10 minutes each time.

- Immediately incubate the filter with secondary antibody.

- Add 10 ml of phosphate-free, azide-free solution (150 mM NaCl, 50 mM Tris-HCl, 5% nonfat dry milk pH 7.5). Phosphate-free, azide-free blocking solution (pH 7.5, 1000 ml) is shown in Table 9.

- Add 0.005 ml of secondary antibody solution (1:2000).

- Incubate 1 – 2 hours at room temperature with gentle agitation.

- Discard secondary and wash with 150 mM NaCl, 50 mM Tris-HCl (pH 7.5) (phosphate-free, azide-free solution) for 3 times for 10 minutes each time.

- Alkaline phosphatase stain:

- Add 5 ml of the substrate 5-brono-4chloro-3-indolyl phosphate/nitro blue tetrazolium (BCIP/NBT) solution (Sigma).

- Observe the filter for the blue color on the filter (about 20 minutes).

- Discard BCIP/NBT solution when the bands are clear (about 20 minutes).

- Immediately stop the enzymatic reaction by add water.

- Cover the filter with plastic membrane and keep the filter.

- Analyze the blue bands and compare the color.

The half-life of a 0.02 mM aqueous solution of PMSF is about 35 minutes, at 8.0 pH. PMSF is usually stored as a 10 mM or 100 mM stock solution (1.74 or 17.4 mg/ml in isopropanol) at –20oC.

1X SDS gel-loading buffer lacking dithiothreitol can be stored at room temperature. Dithiothreitol should then be added, just before the buffer is used, from a 1 M stock (Dissolve 3.09 g of dithiothreitol in 20 ml of 0.01 M sodium acetate (pH 5.2). Sterilize by filtration. Dispense into 1-ml aliquots and store at –20oC).

Table 1. Extract buffer for Western blotting

50 mM Tris-HCl (pH 8.0) or 50 mM HEPES (pH 7.0)

150 mM NaCl

0.02% sodium azide

0.1% SDS

0.1 mg/ml phenylmethylsulfonyl fluoride (PMSF)

0.001 mg/ml aprotinin

1% Nonidet P-40 (NP-40) or 1% Triton X-100.

Table 2. 12% SDS gel preparation reagent amount (l)

Separating gel (%12)	Stacking gel (4%)

Water		1672			3020

Tris-HCl	1250 (1.5 M, pH 8.8)	1250 (0.5 M, pH 6.8)

SDS (10%)	50			50

Acr-Bis (30%)	2000			650

AP		25			25

# TEMED		3			5

Sum		5000			5000

(AP is ammonium persulfate)

Table 3. Optional 12% SDS gel preparation reagent amount (l)

Separating gel (%12)	Stacking gel (4%)

Water		3344			3020

Tris-HCl	2500 (1.5 M, pH 8.8)	1250 (0.5 M, pH 6.8)

SDS (10%)	100			50

Acr-Bis (30%)	4000			650

AP		50			25

# TEMED		6			5

Sum		10000			5000

(AP is Ammonium persulfate)

Table 4. 2 x SDS gel-loading buffer for Western blotting

100 mM Tris-HCl (pH 6.8)

200 mM dithiothreitol

4% SDS

0.2% bromophenol blue

0.2% glycerol

Table 5. Western blotting transfer buffer (1000 ml)

| Tris | 48 mM | 5.814 g |
| --- | --- | --- |
| Glycine | 39 mM | 2.928 g |
| SDS | 0.037% | 0.37 g |
| Methanol | 20% | 200 ml |

SDS: 3.7 ml of 10% SDS

Table 6. Western blotting blocking solution

| Blocking solution, 100 ml, in PBS |  |
| --- | --- |
| Nonfat dried milk | 5% |
| Antifoam A | 0.01% |
| Sodium azide | 0.02% |

Sodium azide: 1 ml of 2% solution

Table 7. Phosphate-buffered saline (PBS), pH 7.4, 1000 ml

| NaCl | 8 g |
| --- | --- |
| KCl | 0.2 g |
| Na2HPO4 | 1.44 g |
| KH2PO4 | 0.24 g |
| Adjust to pH 7.4 with HCl |  |

Table 8. Blocking solution

| Blocking solution, 10 ml, in PBS (pH 7.4) |  |  |
| --- | --- | --- |
| Nonfat dried milk |  | 5% |
| Antifoam A |  | 0.01% |
| Sodium azide |  | 0.02% |

Table 9. Phosphate-free, azide-free blocking solution (pH 7.5, 1000 ml)

| NaCl | 150 mM | 8.766 g |
| --- | --- | --- |
| Tris-HCl (pH 7.5) | 50 mM | 6.057 g |
| 12 N HCl |  | about 3.35 ml |
| Nonfat dried milk | 5% (w/v) |  |

- Overall of Western blotting solutions (Table 10)

Table 10. Overall table of Western blotting solutions

Tissue Extract buffer, 100 ml

50 mM Tris-HCl (pH 8.0), 0.6 g (Or 50 mM HEPES (pH 7.0), 1.19 g)

150 mM NaCl, 0.88 g

0.02% sodium azide, 0.02 g

0.1% SDS, 0.1 g

0.1 mg/ml phenylmethylsulfonyl fluoride (PMSF), 0.01 g

0.001 mg/ml aprotinin, 0.1 mg

1% Nonidet P-40 (NP-40), 1 ml (Or 1% Triton X-100, 1 ml)

2 X SDS gel-loading buffer, 100 ml

100 mM Tris-HCl (pH 6.8), 1.21 g

200 mM dithiothreitol

4% SDS, 0.4 g

0.2% bromophenol blue, 0.2 g

20% glycerol, 20 ml

1.5 M Tris-HCl, pH 8.8, 300 ml

Tris, 54.5 g

HCl, 12 N, 6.375 ml

0.5 M Tris-HCl, pH 6.8, 300 ml

Tris, 18.17 g

HCl, 12 N, 11.5 ml

SDS 5x Running Buffer, pH 8.3, 1000 ml

Tris, 125 mM, 15.14 g

Glycine, 1.25 M, 93.84 g

SDS, 0.50%, 5 g

Western Transfer Buffer, 1000 ml

Tris, 48 mM, 5.814 g

Glycine, 39 mM, 2.928 g

SDS, 0.04%, 0.37 g, 3.7 ml of 10% SDS

Methanol, 20%, 200 ml

Blocking solution, 100 ml, in 100 ml phosphate-buffered saline (PBS, pH 7.4)

Nonfat dried milk, 5%, 5 g

Antifoam A, 0.01%, 10 ml

Sodium azide, 0.02%, 20 mg

Phosphate-buffered saline (PBS), pH 7.4, 1000 ml (adjust to pH 7.4 with HCl)

NaCl, 8 g

KCl, 0.2 g

Na2HPO4, 1.44 g

KH2PO4, 0.24 g

Phosphate-free, azide-free blocking solution, 1000 ml (adjust pH with 12 N HCl about 3.35 ml)

150 mM NaCl, 8.766 g

50 mM Tris-HCl (pH 7.5), 6.057 g

5% (w/v) nonfat dried milk

Correspondence to:

Hongbao Ma

138 Service Road

B410 Clinical Center

Michigan State University

East Lansing, MI 48824

Telephone: 517-432-0623

Email: hongbao@msu.edu

### References

- Dechend R, Homuth V, Wallukat G, Muller DN, Krause M, Dudenhausen J, Haller H, Luft FC. Agonistic antibodies directed at the angiotensin II, AT1 receptor in preeclampsia. J Soc Gynecol Investig. 2006;13(2):79-86.

- Knudson JD, Rogers PA, Dincer UD, Bratz IN, Araiza AG, Dick GM, Tune JD. Coronary Vasomotor Reactivity to Endothelin-1 in the Prediabetic Metabolic Syndrome. Microcirculation. 2006;13(3):209-218.

- Kurien BT, Scofield RH. Western blotting. Methods. 2006;38(4):283-93.

- Ma H. The Structure and Function of Metallothionein. Ph.D. Dissertation of Peking University. 1994:37-46.

- Ma H, Ruiping Huang, George S. Abela. Heat shock protein 70 expression is reduced in rat myocardium following transmyocardial laser revascularization. Journal of Investigative Medicine 2004;52:36.

- Peter-Katalinic J. Methods in enzymology: O-glycosylation of proteins. Methods Enzymol. 2005;405:139-71.

- Sakudo A, Suganuma Y, Kobayashi T, Onodera T, Ikuta K. Near-infrared spectroscopy: promising diagnostic tool for viral infections. Biochem Biophys Res Commun. 2006;341(2):279-84.

- Westermeier R, Marouga R. Protein detection methods in proteomics research. Biosci Rep. 2005;25(1-2):19-32.